- Purpose:

  - The payload dispenser is a cylindrical housing that attaches to a winch to dispense the individual water bottle payloads when close to the ground.

- Description:

  - There are three main parts to the payload dispenser: the housing body, roof, and floor. Inside the housing body are four compartments. One compartment holds an ultrasonic sensor to detect when the housing is close to the ground. The other three have circular cutouts to hold each payload. The housing floor has a circular cutout normally positioned under the ultrasonic sensor. When the ultrasonic sensor detects that the housing is close to the ground, a servo rotates the body to position a payload over the floor cutout, then rotates back before the winch is lowered again. The housing roof and floor are fixed together, with the winch fishing line tied to four points on the roof in a four-leg sling configuration. All three parts are clamped together with a bolt.

- Need:

  - For the payload housing to fit under the aircraft, we will need the airframe to incorporate a cutout in the belly with dimensions of 6.62 inches in diameter by 6.6 inches in depth. It will be at most 3 pounds heavy including all payloads.