- Measure the atmospheric pressure using the barometer and temperature  using the thermometer. Record observed mercury height and temperature.

- Clear the wind tunnel test section and clear the space around the intake and exit of the wind tunnel.

- Measure the atmospheric pressure using the barometer and temperature  using the thermometer.

- Locate the LVDT and switch it on.

- Open the ME30801 folder on the desktop and open the MATLAB App “Force measurement Dynamometer”

- Locate the “Drag Scaling Factor” and “Lift Scaling Factor” that can be found in the bottom-right corner of the MATLAB interface and record them in the Experiment Raw Data Sheet.

- Click the “Run” button located in the top-right corner to initialize the program.

- Save the txt file in your Purdue Home Directory and label it as “Team065_open_lab_test_stand_only”

- Press “Alt + Tab” to go back to the MATLAB App window.

- Enter “0” in the white frequency box in the MATLAB interface, located in the bottom-left box of the interface.

- Press “Scan” and collect data.

- Turn on the wind tunnel

- Set the frequency to 30 Hz by slowly rotating the VFD frequency dial.

- Enter “30” in the white frequency box in the MATLAB interface, located in the bottom-left box of the interface.

- Allow the wind tunnel to run for 10 seconds.

- Press “Scan” and collect data.

- Increase the frequency by 5 Hz by slowly rotating the VFD frequency dial and enter
 the new frequency value in the white box of the MATLAB app interface.

- Allow the wind tunnel to run for 10 seconds.

- Press “Scan” and collect data.

- Repeat steps 16-19 until 60 Hz is reached

- Set the frequency to 0 Hz by slowly rotating the VFD frequency dial.

- Close both the MATLAB app and MATLAB window.

- Turn off the wind tunnel.

- Insert the 180 degree bend test model into the wind tunnel positioning it such that the nut near the front of the model protrudes as little as possible.

- Move the floor to just below the test model just that the feet of the model will not touch the floor if the model vibrates.

- Open the ME30801 folder on the desktop and open the “Force Measurement
 Dynamometer” MATLAB App.

- Click the “Run” button located in the top right corner to initialize the program.

- Save the txt file in your Purdue Home Directory and label it as “Team065_open_lab_180” (replace X with your team number).

- Repeat steps 10-23

- Remove the 180° bend test model and insert the 165° bend test model

- Open the ME30801 folder on the desktop and open the “Force Measurement
 Dynamometer” MATLAB App

- Click the “Run” button located in the top-right corner to initialize the program.

- Save the txt file in your Purdue Home Directory and label it as
 “Team065_open_lab_165”

- Press “Alt + Tab” to go back to the MATLAB App window.

- Repeat steps 10-23

- Remove the 165° bend test model and insert the 150° bend test model

- Open the ME30801 folder on the desktop and open the “Force Measurement
 Dynamometer” MATLAB App

- Click the “Run” button located in the top-right corner to initialize the program.

- Save the txt file in your Purdue Home Directory and label it as
 “Team065_open_lab_150”

- Press “Alt + Tab” to go back to the MATLAB App window.

- Repeat steps 10-23

- Email all data files to teammates.